Pozyx Setup Notes – Single Tag

## Step 1: Place 4x Anchors in desired locations

- NOTE: 4x Anchors are required for 3-D positioning

- Position / locate Anchors in a space to define a cubic volume

- Set Anchor heights (‘z’ position) alternating high / low (odiagonally opposite)

Tag

Anchors

Anchors

- One Anchor must be connected to your computer / laptop via USB cable.  For ease of use, this Anchor can be defined as the point of origin

  - Point of origin anchor has position (0, 0, z)

    - ‘z’ value is set to height of anchor (height is always measured from ground / floor)

- Measure / record each remaining anchor’s position (x, y, z) relative to the point of origin

  - NOTES:

    - ‘x, y’ values are set relative to point of origin

    - ‘z’ value is set to height of anchor (height is always measured from ground / floor)

    - Take measurement from Centre-Top of Anchor

      - where UWB chip is located on board

    - Assign coordinate axes using right hand grip rule

    - Some positions can be negative

- Connect remaining 3x Anchors (remotes) to power outlets and switch on

## Step 2: Required Downloads / Installations

- (Optional) Install Unity3d (2018.2.18f1 or later)

  - https://unity3d.com/

- Install ST’s virtual COM driver:

  - Google Drive link:

    - https://drive.google.com/file/d/1f2j_OHPpYvawyQaeYCNG52dGGsCwoMNI/view?usp=sharing

  - NOTE: This driver is required, even for Windows 10.

- Download the Pozyx Single-Tag Positioning Application:

  - Google Drive link:

    - https://drive.google.com/file/d/1rpPslMHY0bWWWzUqmdbuixaF3MZqA5pb/view?usp=sharing

  - NOTE: this application is used for a (single) Pozyx tag positioning / orientation

  - NOTE: this application may be used for either:

    - Single Tag positioning / orientation

    - Multiple Tag positioning / orientation

  - See Step 4 (below) for additional information

## Step 3: Pozyx Tag(s)

- Connect one (or more) Pozyx Tags to a power source (ie. 9-volt Battery or USB-port / electrical outlet)

- OPTIONS:

  - 9v to 2.1mm DC plug cable (Figure 3a)

    - NOTE: 2.1mm to 2.5mm DC plug adapter required

  - 9v to micro USB (phone charger unit) (Figure 3b)

Figure 3a: Pozyx Tag connected to 9v battery via DC plug cable and adapter

Figure 3b: Pozyx Tag connected to micro USB (phone charger unit)

## Step 4: Run Pozyx – Single Tag Positioning Application (Python-based)

- Download pozyxSingleTag.zip and unzip contents to a user-designated folder on PC

  - Google Drive link:

    - https://drive.google.com/file/d/1rpPslMHY0bWWWzUqmdbuixaF3MZqA5pb/view?usp=sharing

    - Ignore any warnings

  - Contents of file:

    - pozyxSingleTag.exe	- application to run Pozyx (single) tag positioning

    - tagIDs.txt		- list of available tags (Do not modify)

    - anchors.txt		- list of anchor IDs and user-defined co-ordinates

- Open ‘anchors.txt’ file in Notepad or any text file editor

  - Edit / update anchor (x, y, z) co-ordinates for each anchor (as required)

    - Using measured values from Step 1

    - NOTE: this file only needs to be modified / updated if the Pozyx Anchors’ positions are changed

Figure 4: Contents of ‘anchors.txt’ file showing anchor IDs and user-definable coordinates

- Run pozyxSingleTag.exe application file (double-click)

  - NOTES:

    - ensure Pozyx Anchors and Tag are connected and powered up prior

    - for first time execution, ignore any Windows security warnings

Figure 5: Ignore Windows Defender warnings: click ‘Run anyway’

    - ‘tagIDs.txt’ and ‘anchors.txt’ files must be kept in same directory as pozyxSingleTag.exe

  - application will execute and a console window will be displayed, showing a user menu

Figure 6: Pozyx Tag selection menu is presented to user

  - select the appropriate Pozyx Tag from the menu

    - eg: 1 to operate Pozyx Tag #0x0001

  - Pozyx Tag will commence positioning

Figure 7: Sample output of Pozyx positioning system during execution

  - NOTES:

    - this application feeds position (x, y, z) and orientation (p, r, y) into Unity via OSC Protocol

    - this application MUST be run prior to attempting to run the Unity Project

    - anchor co-ordinates (x, y, z) are stored in ‘anchors.txt’ file and should be modified only if anchor(s) are moved into a new location / position (Figure 4)

    - tag IDs (hexadecimal values) are stored in ‘tagIDs.txt’ file and should not be modified

      - corresponding tag ID labels are stuck on to rear of tags (Figure 8)

Figure 8: Pozyx Tag ID

## Step 6: (Optional) Run Unity-Project: PozyxUnity - SingleTag

- Download and unzip the Unity project ‘PozyxUnity – SingleTag’ to a user-designated folder:

  - Google Drive link:

    - https://drive.google.com/file/d/1RP4hsVZryN91K9Id88VVdQQ0z3N2tTY9/view?usp=sharing

    - Ignore any warnings

- Open Unity project ‘PozyxUnity – SingleTag’ in Unity3D application

  - (Optional) Select ‘Pozyx Tag’ game object (left menu) and enter desired position and orientation smoothing values (right menu: Position (Script))

    - NOTES:

      - Smoothing values can be individually set for both position and / or orientation motion to reduce jerkiness

      - Values are set from:

        - 1 = No smoothing

        - 2 or greater = increased smoothing

      - Higher smoothing values increases motion latency / delay

Figure 9: Unity project interface

- While ‘pozyxSingleTag.exe’ is running (see Step 5), press ‘PLAY’ on Unity interface

  - NOTES:

    - ‘pozyxSingleTag.exe’ must be running prior to playing the Unity project

      - Data is fed from Pozyx, into the application and then streamed into Unity via Open Sound Control (OSC) protocol

    - STOP playing Unity project before stopping / closing Python script

      - To avoid a continuous loop / Unity appears to hang

      - If this does occur, simply re-run python script

Figure 10: Running ‘PozyxUnity – SingleTag’ Unity Project

## Additional Notes

- Accuracy of the Poxyx Tag(s) may diminish as battery life deteriorates

## Source Code

- All source code (Python) files are available on Google Drive:

  - Link: https://drive.google.com/drive/folders/1fSZsK0Kfqo2mBGzqIN_doj5CTJE9IFVQ?usp=sharing

- Original Pozyx source code (Python) files are available on Google Drive:

  - Link: https://drive.google.com/drive/folders/140HnsQ3DgbZL-wxljcpPInDfQ488XVY4?usp=sharing

  - Github: https://github.com/pozyxLabs/Pozyx-Python-library

- Install Python 3.7 (or later)

  - https://www.python.org/downloads/windows/

  - Install 32-bit or 64-bit (as required)

  - NOTES:

    - make sure “Add Python 3.X to PATH” is checked / activated

(refer to Figure 1)

Figure 1: Add Python to PATH

    - (optional) Disable PATH length limit

(refer to image)

Figure 2: Disable Path length limit (optional)

- Install Pozyx and OSC Python libraries:

  - Download pozyxInstallPackages.zip, unzip and run pozyxInstallPackages.exe

    - Ignore any virus warnings

  - Google Drive link:

https://drive.google.com/open?id=1DJPwCxmspRbBYAJbxxrxKIAEUOH08UbV

  - NOTE: if there are problems downloading / installing packages using application, individual libraries can also be installed manually:

    - Open a “COMMAND-PROMPT” (DOS prompt) in Windows:

      - Press ‘Windows’ + ‘R’ keys

      - Enter cmd in RUN dialog

      - Press ENTER or click OK

    - At C:\> prompt, enter the following commands (individually):

    - pip install pypozyx

    - pip install python-osc

    - pip install requests

    - pip install pyserial

  - For additional information on Pozyx and application development: refer to https://www.pozyx.io/Documentation/Tutorials/getting_started/Python