Pozyx Setup Notes

## Step 1: Place 4x Anchors in desired locations

- NOTE: 4x Anchors are required for 3-D positioning

- Position / locate Anchors in a space to define a cubic volume

- Set Anchor heights (‘z’ position) alternating high / low (or diagonally opposite as shown below)

Tag

Anchors

Anchors

- One Pozyx device must be connected to your computer / laptop via USB cable for the system to work.  For ease of use, connect an Anchor and define it as the point of origin

  - Point of origin anchor has position (0, 0, z)

    - ‘z’ value is set to height of anchor (height is always measured from ground / floor)

  - Note: an alternative is to utilise a spare Tag as a surrogate Anchor / Master controller.  Simply remove the T/A Jumper from the tag’s board and connect the device to your computer / laptop (via USB cable).

T/A Jumper removed:

(Tag becomes Anchor)

    - Device will power on and automatically communicate with other remote Pozyx devices (Anchors and Tag(s)).

      - Plug-and-play option – no file changes are required!

    - (Optional): Utilise as a true Anchor.  Measure “new” anchor’s position relative to origin and add information to ‘anchors.txt’ file.

      - > 4 anchors = increased accuracy in 3-D

- Measure / record each remaining anchor’s position (x, y, z) relative to the point of origin

  - NOTES:

    - ‘x, y’ values are set relative to point of origin

    - ‘z’ value is set to height of anchor (height is always measured from ground / floor)

    - Take measurement from Centre-Top of Anchor

      - where UWB chip is located on board

    - Assign coordinate axes using right hand grip rule

    - Some positions can be negative

- Connect remaining 3x Anchors (remotes) to power outlets and switch on.

## Step 2: Required Downloads / Installations

- (Optional) Install Unity3d (2018.2.18f1 or later)

  - https://unity3d.com/

- Install ST’s virtual COM driver:

  - Google Drive link:

    - https://drive.google.com/file/d/1f2j_OHPpYvawyQaeYCNG52dGGsCwoMNI/view?usp=sharing

  - NOTE: This driver is required, even for Windows 10.

- Download the Pozyx Positioning Application:

  - Google Drive link:

    - https://drive.google.com/file/d/15kPOfWOS7a3x8w7s4qwi18ec-KUv8-FO/view?usp=sharing

  - NOTE: this application may be used for either:

    - Single Tag positioning / orientation

    - Multiple Tag positioning / orientation

  - See Step 4 (below) for additional information

## Step 3: Pozyx Tag(s)

- Connect one (or more) Pozyx Tags to a remote power source (ie. 9-volt Battery or USB-port / electrical outlet)

- OPTIONS:

  - 9v to 2.1mm DC plug cable (Figure 3a)

    - NOTE: 2.1mm to 2.5mm DC plug adapter required

  - 9v to micro USB (phone charger unit) (Figure 3b)

Figure 3a: Pozyx Tag connected to 9v battery via DC plug cable and adapter

Figure 3b: Pozyx Tag connected to micro USB (phone charger unit)

## Step 4: Run Pozyx Positioning Application

- Download pozyxPositioning.zip and unzip contents to a user-designated folder on PC

  - Google Drive link:

    - https://drive.google.com/file/d/15kPOfWOS7a3x8w7s4qwi18ec-KUv8-FO/view?usp=sharing

    - Ignore any warnings

  - Contents of file:

    - pozyxPositioning.exe	- application to run Pozyx (single) tag positioning

    - tagIDs.txt		- list of available tags (Do not modify)

    - anchors.txt		- list of anchor IDs and user-defined co-ordinates

    - ReadMe.txt		- information on how to setup / run the application

- Open ‘anchors.txt’ file in Notepad or any text file editor

  - Edit / update anchor (x, y, z) co-ordinates for each anchor (as required)

    - Using measured values from Step 1

    - NOTE: this file only needs to be modified / updated if the Pozyx Anchors’ positions are changed

Figure 4: Contents of ‘anchors.txt’ file showing anchor IDs and user-definable coordinates

- Run pozyxPositioning.exe application file (double-click)

  - NOTES:

    - ensure Pozyx Anchors and Tag are connected and powered up prior

    - for first time execution, ignore any Windows security warnings

Figure 5: Ignore Windows Defender warnings: click ‘Run anyway’

    - ‘tagIDs.txt’ and ‘anchors.txt’ files must be kept in same directory as pozyxPositioning.exe

  - application will execute and a console window will be displayed, showing a user menu

Figure 6: Pozyx Tag selection menu is presented to user

  - select the appropriate Pozyx Tag(s) from the menu

    - Single Tag Operation:

      - Example:

        - Press ‘1’, then ‘ENTER’ to select Pozyx Tag #0x0001

        - Press ‘r’, then ‘ENTER’ to start positioning operation

    - Multiple Tag Operation:

      - Example:

        - Press ‘2’, then ‘ENTER’ to select Pozyx Tag# 0x0002

        - Press ‘3’, then ‘ENTER’ to select Pozyx Tag# 0x0003

        - Press ‘r’, then ‘ENTER’ to start positioning operation

  - Pozyx Tag will commence positioning

Figure 7: Sample output of Pozyx positioning system during execution

  - NOTES:

    - this application feeds position (x, y, z) and orientation (p, r, y) into Unity via OSC Protocol on:

      - IP Address: 127.0.0.1	[local host]

      - Port: 8888

    - this application MUST be run prior to attempting to run the Unity Project

    - anchor co-ordinates (x, y, z) are stored in ‘anchors.txt’ file and should be modified only if anchor(s) are moved into a new location / position (Figure 4)

    - Tag IDs (hexadecimal values) are stored in ‘tagIDs.txt’ file and should not be modified

      - corresponding Tag ID labels are stuck on to rear of Tags (Figure 8)

      - corresponding Anchor ID labels are stuck on to top of Anchors

Figure 8: Pozyx Tag ID

## Step 6a: (Optional) Run Unity-Project: PozyxUnity - SingleTag

- Download and unzip the Unity project ‘PozyxUnity – SingleTag’ to a user-designated folder:

  - Google Drive link:

    - https://drive.google.com/open?id=1MdhZ8oSKFlxqlxmXYsQcvGXuPbf__9Jn

    - Ignore any warnings

Figure 9: Unity Project interface

- Open Unity project ‘PozyxUnity – SingleTag’ in Unity3D application (see Step 1)

  - (Optional) Select ‘Pozyx Tag’ game object (left menu) and enter desired position and orientation smoothing values (right menu: Position (Script))

Figure 10: PozyxTag (GameObejct) – Smoothing Settings

    - NOTES:

      - Smoothing values can be individually set for both position and / or orientation motion to reduce jerkiness

      - Values are set from:

        - 1 = No smoothing

        - 2 or greater = increased smoothing

      - Higher smoothing values increases motion latency / delay

- While ‘pozyxPositioning.exe’ is running (see Step 5), press ‘PLAY’ on Unity interface

  - NOTES:

    - ‘pozyxPositioning.exe’ must be running prior to playing the Unity project

      - Data is fed from Pozyx, into the application and then streamed into Unity via Open Sound Control (OSC) protocol

    - STOP playing Unity project before stopping / closing Python script

      - To avoid a continuous loop / Unity appears to hang

      - If this does occur, simply re-run python script then STOP Unity project

Figure 11: Running ‘PozyxUnity – SingleTag’ Unity Project

## Step 6b: (Optional) Run Unity-Project: PozyxUnity – MultiTag

- Download and unzip the Unity project ‘PozyxUnity – MultiTag’ to a user-designated folder:

  - Google Drive link:

    - Ignore any warnings

Figure 9: Unity Project interface

- Open Unity project ‘PozyxUnity – MultiTag’ in Unity3D application (see Step 1)

  - Activate / De-activate desired tag(s) to use

    - Select appropriate ‘Pozyx Tag’ game object (left menu)

    - Click ‘GameObject’ from top menu

    - Click ‘Toggle Active State’ from drop down menu

Figure 10: PozyxTag (GameObejct) toggling Active / Inactive

    - NOTES:

      - Ensure that only Tags in use are set to active

Eg: if using Tag 1 and Tag 3, these should be active, with the remaining Tag game objects inactive

      - Failure to do so correctly may result in Unity freezing / errors

  - Smoothing values can be individually set for both position and / or orientation motion to reduce jerkiness

    - Values are set from:

      - 1 = No smoothing

      - 2 or greater = increased smoothing

      - Higher smoothing values increases motion latency / delay

- While ‘pozyxPositioning.exe’ is running (see Step 5), press ‘PLAY’ on Unity interface

  - NOTES:

    - ‘pozyxPositioning.exe’ must be running prior to playing the Unity project and applicable tags selected

      - Tags have been named to match

        - eg: Pozyx Tag 0x0001 is PozyxTag_01 in Unity

      - Data is fed from Pozyx, into the application and then streamed into Unity via Open Sound Control (OSC) protocol

    - STOP playing Unity project before stopping / closing Python script

      - To avoid a continuous loop / Unity appears to hang

      - If this does occur, simply re-run python script then STOP Unity project

Figure 11: Running ‘PozyxUnity – SingleTag’ Unity Project

## Additional Notes

- Accuracy of the Poxyx Tag(s) may diminish as battery life deteriorates

## Source Code

- All source code (Python) files are available on Google Drive:

  - Link: https://drive.google.com/drive/folders/1fSZsK0Kfqo2mBGzqIN_doj5CTJE9IFVQ?usp=sharing

- Original Pozyx source code (Python) files are available on Google Drive:

  - Link: https://drive.google.com/drive/folders/140HnsQ3DgbZL-wxljcpPInDfQ488XVY4?usp=sharing

  - Github: https://github.com/pozyxLabs/Pozyx-Python-library

- Install Python 3.7 (or later)

  - https://www.python.org/downloads/windows/

  - Install 32-bit or 64-bit (as required)

  - NOTES:

    - make sure “Add Python 3.X to PATH” is checked / activated

(refer to Figure 1)

Figure 1: Add Python to PATH

    - (optional) Disable PATH length limit

(refer to image)

Figure 2: Disable Path length limit (optional)

- Install Pozyx and OSC Python libraries:

  - Download pozyxInstallPackages.zip, unzip and run pozyxInstallPackages.exe

    - Ignore any virus warnings

  - Google Drive link:

https://drive.google.com/open?id=1DJPwCxmspRbBYAJbxxrxKIAEUOH08UbV

  - NOTE: if there are problems downloading / installing packages using application, individual libraries can also be installed manually:

    - Open a “COMMAND-PROMPT” (DOS prompt) in Windows:

      - Press ‘Windows’ + ‘R’ keys

      - Enter cmd in RUN dialog

      - Press ENTER or click OK

    - At C:\> prompt, enter the following commands (individually):

    - pip install pypozyx

    - pip install python-osc

    - pip install requests

    - pip install pyserial

  - For additional information on Pozyx and application development: refer to https://www.pozyx.io/Documentation/Tutorials/getting_started/Python